# HP3932C 32 in 1 IP QAM Modulator

## Product Overview

HP3932C 32 in 1 IP  QAM  modulator  is  the latest  generational  Mux-scrambling-modulating all-in-one device developed by CATCAST. It supports maximum 1024 IP input through the 3 GE ports and 32 multiplexing scrambling QAM (DVB-C) modulating channels with non-adjacent carriers (50MHz~960MHz) output through the RF output interface. Its pay-as-you-grow modular design and flexible configuration are making it extremely scalable, very reliable with high performance, all of which are adaptable to newly generation CATV broadcasting system.

## Key Features

l  3 GE ports (max 1024 IP in):

Data1 & Data2 bi-directional ports, max 512 IP in, 32 IP out

Data port (located on front panel), max 128 IP in

l  Max 840Mbps for each GE input

l Supports accurate PCR adjusting/CA filtering, PID remapping and PSI/SI editing

l  Supports up to 256 PIDS remapping per channel

l  Support 32 IP output through Data1 & Data2 over UDP/RTP/RTSP

l 32 non-adjacent QAM carriers output, compliant to DVB-C (EN 300 429) and ITU-T J.83 A/B

l  Supports RS (204,188) encoding

l  Support Web-based Network management

Inner Principle Chart

Carrier Setting Illustration

System Connection Sample (IP Based)

Specifications

| Input | Input | Max 1024 IP input through 3 (front-panel Data port, Data 1 and Data 2) 100/1000M Ethernet Port (SFP interface optional). Each Data1 or Data 2 port can input maximum 512 IP, while front-panel Data port can input maximum 128 IP |  |
| --- | --- | --- | --- |
|  | Transport Protocol | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |  |
|  | Transmission Rate | Max 840Mbps for each GE input |  |
| Mux | Input Channel | 1024 |  |
|  | Output Channel | 32 |  |
|  | Max PIDs | 256 per channel |  |
|  | Functions | PID remapping(auto/manually optional) |  |
|  |  | PCR accurate adjusting |  |
|  |  | PSI/SI table automatically generating |  |
| Scrambling Parameters | Max simulscrypt CA | 4 |  |
|  | Scramble Standard | ETR289, ETSI 101 197, ETSI 103 197 |  |
|  | Connection | Local/remote connection |  |
| Modulation Parameters | DVB-C Modulator Section | J.83A | Constellation : 16/32/64/128/256QAM |
|  |  |  | Bandwidth :8M |
|  |  | J.83B | Constellation : 64QAM/ 256QAM |
|  |  |  | Bandwidth :6M |
|  | QAM Channel | 32 non-adjacent carrier |  |
|  | Modulation Standard | EN300 429/ITU-T J.83A/B(DVB-C) |  |
|  | Symbol Rate | 5.0~7.0Msps, 1ksps stepping |  |
|  | Constellation | 16, 32, 64 , 128, 256QAM |  |
|  | FEC | RS (204, 188) |  |
| RF Output | Interface | 1 F typed output port for 32 carriers, 75Ω |  |
|  | RF Range | 50~960MHz, 1kHz stepping |  |
|  | Output Level | -20dBm~+10dBm(87~117dbµV), 0.1dB stepping |  |
|  | MER | ≥ 40dB |  |
| TS output | 32 IP output over UDP/RTP/RTSP, unicast/multicast, 2 (Data1& Data2) 100/1000M Ethernet Ports |  |  |
| System | Network management software (NMS) supporting |  |  |
| General | Demission | 420mm×440mm×44.5mm (WxLxH) |  |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |  |
|  | Power Supply | AC 100V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |  |